HC12 Interrupts using the HC12 Real Time Clock

Process for Setting up and enabling the Real Time Clock - Interrupt Service Routine

- Setup RAM memory vector to ISR using SetUserVector()

- Program the RTICL for the Interrupt Clock Interval

- Enable the RTIE bit in CRGINT

- In the RTISR, clear the RTIF by writing a 1 to it.

- Finally enable global interrupts by clearing the Interrupt Mask bit in the CCR