CSC 202						Name:________________________

Exam #1		OPEN BOOK – NO CALCULATORS

Directions: Do all work on these sheets.  Organize your work in a neat and orderly fashion.

1. [10]  Convert the signed binary number 1111 01012   to decimal form with a + or a - sign.

The msb indicates this number is negative.   It is F5(16).  Lets 1’s complement and add 1 to

Negate the number  FF-F5=0A+1=0B or the number is -11(10)

2. [10] Convert 0ABC to decimal.

This is A*16^2 + B*16^1 + C*16^0 or 10*256 + 11*16 + 12 = 2560 + 176 + 12 = 2748

3. [10]  Convert decimal –4368 to its signed bit, 16 bit Hexadecimal equivalent.

1st let’s convert 4368 to hex  4368 – 4096 = 272 thus far we have 1_ _ _ hex

272 – 256 = 16 thus we have 11 _ _ hex

16 – 16 = 0 so finally we have 1110 hex     now lets take the 2s complement to negate the number

FFFF – 1110 = EEEF + 1 = EEF0

4. [20] What are the minimum number of bits required to store the following signed decimal numbers in 2’s complement form?

a)  -64		b)  -4		c) 512		d) 5		e) 65535

7		3		11		4		17

5. [50]  Show the hexadecimal contents of  the listed memory locations after running  the program.

ORG	$1000			$1000 _CE_	$1007 _20_	$100E _00_

START	LDX	#DATA			$1001 _20_	$1008 _00_	$100F _6C_

STX	DATA+4		$1002 _00_	$1009 _BB_

LDD	DATA			$1003 _0C_	$100A _20_	$1011 _CE_

ADDA	DATA			$1004 _20_	$100B _00_	$1012 _10_

STAA	DATA			$1005 _04_	$100C _7A	$1013 _00_

STD	2,+X			$1006 _FC_	$100D _20_	$1014 _6B_

LDX	#START						$1015 _03

STAB	3,X							$1016 _3F_

SWI

ORG	$2000			$2000 _02_	$2004 _??_

DATA   DB	1,12,0,10		$2001 _0C_	$2005 _??_

$2002 _02_	$2006 _??_

$2003 _0C_