IASM12 USERS MANUAL

LICENSE, LIABILITY, and ACKNOWLEDGEMENTS ............  1

IASM12 - The Integrated Assembler ...................  2

INSTALLATION ........................................  4

HOTKEYS .............................................  6

MENU ................................................  7

HELP ................................................  9

EDITOR .............................................. 10

Entering and Editing Test ......................... 10

The Editing Screen ................................ 10

Tabs .............................................. 11

Windows ........................................... 12

Cursor Movement Commands .......................... 13

Insert and Delete Commands ........................ 15

Block Commands .................................... 16

Miscellaneous Commands ............................ 18

ASSEMBLER ........................................... 20

Labels ............................................ 21

Assembler Directives .............................. 21

Conditional Assembly .............................. 23

Includes .......................................... 24

Macros ............................................ 25

Constants ......................................... 26

Opcodes ........................................... 26

Operands .......................................... 27

Comments .......................................... 27

Pseudo-Ops ........................................ 28

Listing Directives ................................ 29

Listing File ...................................... 30

Object files ...................................... 30

Map files ......................................... 31

Errors ............................................ 32

COMMUNICATIONS ...................................... 34

INSTRUCTION SET ..................................... 36

INSTRUCTION FORMATS ................................. 40

ADDRESSING MODES .................................... 43

Inherent Addressing ............................... 43

Immediate Mode (IMM) .............................. 43

Direct Mode (DIR) ................................. 43

Extended Addressing (EXT) ......................... 43

Indexed 5-BIT Constant Offset (IDX) ............... 44

Indexed Auto Pre/Post Decrement/Increment (IDX) ... 44

Indexed Accumulator Offest (IDX) .................. 44

Indexed 9-Bit Constant Offset (IDX1) .............. 44

Indexed 16-Bit Constant Offset (IDX2) ............. 45

Indexed 16-Bit Constant Indirect ([IDX2]) ......... 45

Indexed Auto Pre/Post Decrement/Increment (IDX) ... 45

Indexed Accumulator Offest (IDX) .................. 46

Indexed D-Indirect ([D,IDX]) ...................... 46

Multiple Addressing Modes ......................... 46

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - Index

LICENSE

LICENSE, LIABILITY, and ACKNOWLEDGEMENTS

P&E Microcomputer Systems License Agreement:

The IASM12 license is a "single user" license. You may:

1. Personally use the program on any machine;

2. Copy the program into any machine-readable or printed

form for your personal use as backup only.

You may not:

1. Use, copy, modify, or transfer the program, in

whole or in part, except as noted above;

2. De-compile or disassemble the code for the program;

3. Use or place the program on any time sharing or other

interactive system including but not restricted to

workstation networks, bulletin boards or multi-user

systems; 4. Rent or lease this program.

This program may only be used on one computer at any one time.

LIABILITY

P&E Microcomputer System does not assume any liability for the

use of IASM12 beyond the original purchase price of the software.

In no event will P&E Microcomputer Systems be liable to you for

additional damages, including any lost profits, lost savings or

other incidental or consequential damages arising out of the use

of or inability to use this program, even if P&E Microcomputer

Systems, Inc. has been advised of the possibility of such

damages.

ACKNOWLEDGEMENTS Portions of the software are: copyright 1987 by

Borland International copyright 1988 by TurboPower Software

Manufacture's specifications offered in the HELP screens are

reprinted with the manufacture's permission.  See "MANUFACTURER"

within the HELP system.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 1

IASM12

IASM12 - The Integrated Assembler

IASM12 is an editor, cross assembler, and communications package.

All three applications have been blended into a single

environment to greatly simplify your job of writing and debugging

source code. With IASM12 you will be able to easily correct any

syntactical errors in your source code without leaving the

environment. IASM12's integrated editor is a standard source file

editor. The integrated cross assembler is similar to P&E

Microcomputer's past cross assemblers but has been optimized for

the new environment. The built in communication environment

enables you to work with any development system attached to your

PC. Assembled files may be downloaded and tested while the

original source code is edited and assembled.

System Requirements

IASM12 runs under MS-DOS or PC-DOS on an IBM PC, XT, AT or

compatible. It is greatly recommended to have at least 512

Kilobytes of system memory. The communications uses COM1 or

COM2.  If you intend to print from within IASM12, the standard DOS

printer port is used.

System Overview

IASM12 lets you generate standard assembly language source code or

read it in from a disk file. Three output files may be generated.

One is the object file, machine language for the target

processor. The second is a listing file: a copy of the input text

with various annotations such as the machine code, cycle timing,

etc. The third is a map file. This is used by other P&E

Microcomputer software such as simulators and user interfaces.

These files may or may not be generated, depending on the options

the user chooses. If an error is found during assembly of a file,

the line in question is highlighted on the screen, the cursor is

placed near the error, and the user is told of the problem. The

communications window may be opened at anytime and the code

tested on a development board.

Getting Started

Two files must reside in your working directory or on your DOS

path (see your DOS manual) for the environment to work properly.

One is the main file IASM12.EXE. The other file is the on-screen

help file, IASM12.HLP. This file is only needed if you plan on

using the built-in help system.

To start the environment all you need to type is the command line

:

IASM12 <filename>

where <filename> is an optional file name that will immediately

be loaded into the editor. Note that the editor will, as a

default, name all files with the extension .ASM. If a filename is

not specified, the editor will start with a blank file entitled

"NONAME.ASM". You will be prompted to change it's name upon

saving the file.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 2

NOTE: Any source code used as examples in this manual may or may

not use mnemonics that are acceptable for your version of the

assembler.

If you are using source code that has been written for another

assembler, see the Appendix at the end of this manual.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 3

INSTALLATION

Installation You may choose to run the installation program at

any time and as many times as you wish. The program has four

basic functions. First, it allows you to change any of the editor

commands. Second, it enables you to choose any color scheme you

like. Third, you set up various parameters concerning the

assembler. Fourth, you set up the parameters concerning the

communications.

To invoke the installation program, at the DOS prompt type:

IASMINST IASM12

You will first have the opportunity to modify certain parameters

for the editor. In most cases, hitting Enter will accept the

default value shown. You will be asked for a DOS string to be

executed when F6 is hit during an editing session. This is a

dedicated shell to DOS and may be left blank if desired.

Next you can modify several assembler choices. You will be

prompted for default settings of all the items in the ASSEMBLE

sub-menu. See that section for a full explanation.

Next you can modify the communications parameters. You will be

prompted for default settings of all the items in the COMM

sub-menu. See that section for a full explanation.

At this point you will be prompted for color choices. The screen

shows all possible colors or shades for the current monitor.

Choose whichever colors suit your taste. Use the up and down

arrow keys to choose the attribute you want to set and hit Enter.

Now use the arrow keys to choose a color and select it with the

Enter key. When finished, hit Esc.

You will now be asked if you want fast entry mode for keyboard

configuration. The default entry is NO for random access entry.

These are ways of reconfiguring the editor commands.

Fast Entry

If you choose fast entry, you will receive on line instructions.

Basically, you are shown each command one at a time and can

either leave it the way it is (press Enter), change it using the

Bksp key, clear it by hitting C or restore it by hitting R. If

you specify a new command, terminate it by hitting Enter. If you

wish Enter to be part of the command, first press the Scroll-Lock

key to accept keys literally. Hit it again to go back to the

normal mode. Pressing Esc takes you out of the fast entry mode

and puts you into the random access mode.

Random Access

This mode lets you alter any or all commands in any order.

Instructions are shown at the top of the screen. Use the cursor

keys to move the cursor to the entry you want to change. The

editing functions are the same as described in fast entry, above.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 4

Moving on ...

When you are done making changes you can hit the Esc followed by

either Q or W.

W W will bring up the message "Checking for conflicts ..." If you

have assigned the same sequence to two commands, you will be

given an error message. You can correct the mistake by looking

for highlighted items. You will also get an error message if the

new list of keystrokes is too long. If so, just eliminate some of

the secondary key sequences, or any you will not be using.

Q Q will cause the installation program to move on and not

change any commands.

NOTE: Whenever you assemble a program and a listing file is

generated, the assembly time will be increased. It is

recommended that you set this default to OFF and only turn it on

when you want to generate a listing file. The same is true of the

object file.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 5

HOTKEYS

Hotkeys

You will notice that several hotkeys are shown on the bottom of

the screen display. They are :

F1 : Help   This key will bring up the help; system.

F2 : Save   Hitting F2 will cause the file currently

in the editor to be saved, a backup file

to be made and the cursor returned to the

point just before F2 was selected.

F3 : Load   Hitting F3 will allow the user to load

in a new file. You will be prompted, if

any changes have been made to the current

file, to save the current file and then

asked for the name of the new file.

Hitting ^ F3 (Cntrl F3) will do the same

but the file loaded will be the file

specified on the command line when

IASM12 was entered.

F4 : Assemble   Hitting F4 will start the assembly of

the file currently in the editor. Any

options chosen from the main menu system

will be in effect. Note that only one

window may be open during assembly.

F5 : Exit   Hitting F5 will cause an end to the

editing-assembling session. If any

changes have been made to the current

file, you will be given an opportunity

to save them before returning to DOS.

If you are in a secondary window, F5

will close that window.

F6 : DOS    String Hitting F6 causes a shell to DOS

with the string specified in the

installation program.

F7 : Comm   Hitting F7 will either open the

communications window or, if it is

already open, will make the communications

window the active window.

F9 : DOS shell  Hitting F9 will put you into DOS.

Typing EXIT at the DOS prompt will return

you into IASM12.

F10 : Menu  Hitting F10 will cause the main menu system

to appear.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 6

MENU

Menu

The Menu system contains a highlighted bar along the bottom of

the screen. You can choose a menu ;item by highlighting it with

the cursor keys or entering the highlighted letter within your

choice ('E' in Edit, for example). Hitting Esc; will go back to

the previous menu (if you are within a submenu) or return you to

the editor.

Edit:   This choice will put you back into the editor.

There are no submenus.

File:   This choice will open a submenu with the

following choices :

Save:   The file currently in the editor is

saved, a backup file is made and the

cursor returned to the point just before

the selection.

Load:   Loads in a new file. You will be prompted,

if any changes have been made to the

current file, to save the current file

and then asked for the name of the new file.

Dir:    Shows a directory listing using the mask

prompted for. The default mask is *.asm,

thus showing all files in the current

directory having an extension of .ASM.

If the cursor is placed on a filename and

Enter is pressed, that file will be loaded

into the editor.

Assemble: This choice will open a submenu with the

following choices:

Assemble: Performs the assembly of the file

currently in the editor. Any options

chosen from this submenu will be in

effect. Note that only one window may

be open during assembly.

Listing:  This choice toggles between on and off

indicating whether or not a listing file

(.LST) will be created at assembly time.

Note that assembly time is increased

when listing is turned on.

Object:   This choice toggles between off, hex

and s19 indicating whether or not an

object file will be created at assembly

time and what type it will be. Choose

the type needed by your development

system or PROM programmer. Note that

assembly time is increased when an

object file is created.

Cycle Cntr: This choice toggles between on and

off indicating whether or not instruction

cycle counts will be included in the

listing file.

Debug Map: This choice toggles between on and off

indicating whether or not a map file

(.MAP) will be created at assembly time.

Note that assembly time is increased when

a map file is created. Map files are used

with P&E Microcomputer simulators and

interface software.

Macro:   This choice toggles between view and

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 7

hide. View means that the source code

for macros will be printed in the .LST

file at every macro call. Hide

suppresses this printing.

Include: This choice toggles between view and

hide. View means that the source code

within the INCLUDE files will be printed

in the .LST file. Hide suppresses this

printing.

Communicate: This choice will open a submenu with the

following choices:

Comm:    This choice toggles between COM1 and

COM2, the hardware ports recognized

by the operating system.

Baud:    This choice toggles through the list

of possible baud rates from 110 to 9600.

Parity:  This choice toggles between N for none,

E for even and O for odd.

Length:  This choice toggles between 7 and 8

indicating the number of bits per word

to use.

Stop:    This choice toggles between 1 and 2

indicating the number of stop bits per

word to use.

Help:   Brings up the help menu.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 8

HELP

Help

Hitting F1 from within the editor brings up the menu driven help

system. The initial window displays a list of topics from which

to choose. Using the cursor keys, move the highlight bar over the

topic desired. Note that when there are more topics than the

window shows, the window will scroll with the up and down arrow

keys. Once an item is chosen, hitting Enter will display the

first help page for that item. PgDn and PgUp keys will show the

next or previous pages. Hitting Esc at any time will exit the

current help window going to the previous one, exiting the help

system from the first window.

The help system covers three main areas. The first is the editor

itself and all of the commands are given. If any commands have

been changed via IASMINST, the changes will not be reflected

here. The second area is the assembler. Most assembler commands,

options and structures are explained. Finally there are

microcontroller - microprocessor specific topics. In most cases

this information is from the manufacturer and reprinted with

their permission. P&E Microcomputer has made every effort to

assure the accuracy of this information but assumes no

responsibility for it. See the help topic "MANUFACTURER" for more

information. Also note that the chip information is generally for

the entire family of chips that this specific IASM12 covers, hence

not all of the information may be applicable to your application.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 9

EDITOR

Editor

Entering and Editing Text

Text is entered into the editor much like it is on a typewriter

and most of the keys act the same way. However, there are several

important differences.

1. The cursor is always positioned at the spot where new

text will be entered.

2. There are several delete commands to correct mistakes

quickly and easily.

3. There are several block commands for moving and

copying text.

4. There are Find and Find-and-replace commands.

5. In many cases you can "undo' your last several

commands with the Restore option.

All of the commands are described in the text that follows. For a

quick reference, refer the back cover of this manual.

The Editing Screen

The top line of the screen is set aside as a prompt line.

Messages and instructions are displayed here as are responses to

prompts (see the section "Prompt Editor"). When the first key of

a two key command is entered, it is echoed at the left edge of

the prompt line.

The top line of any editing window is a status line with the

following information:

FILENAME.EXT

Shows the name and extension of the file being edited.

You may specify full path names to the editor but they

will not show up here.

Line n

This is the line number, starting from the start of the

file, that the cursor is currently on.

Col n

The number of the column containing the cursor.

Byte n

The position of the cursor relative to the first

character in the file.

Insert

Shows that the editor is in insert mode (as opposed

to overwrite).

Indent

Shows that the editor is in auto-indent mode.

Tab

Shows that the editor is using fixed tab stops.

Save

This indicates that the file has been modified since

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 10

it was last saved.

Prompt Editor

When IASM12 prompts you for a response to a question, you are

generally presented with a default answer which may be accepted

by hitting Enter. You may cancel the operation by hitting Esc.

When you are entering text at the prompt line, you are using the

prompt editor, a line editor with its own set of commands. Most

are the same as the full editor in IASM12 with the following

additions:

Function        Key Command

Accept entry    Enter or ^M

Abort           Esc

Insert control

character       ^P

==================================================================

TABS

Tabs

There are two tab settings, smart tabs and fixed tabs.

Smart tabs: When smart tabs is enabled, the tab stops are

determined by the previous line. The start of any non-space

sequence (the first letter in a word, for instance) acts as a tab

stop. This is generally the easiest mode to use when entering

source code.

Fixed tabs: Tab stops start at column 9 and then every 8

columns. This can be changed in the installation program to any

number desired. Note that the first tab stop is always in column

(tab size + 1).

With either setting, tabs are translated to spaces

automatically. When a file is read into the editor, all tabs are

expanded to the default tab settings. When a file is written,

spaces will be encoded into tabs if this option is enabled via

IASMINST, the installation program. This will save disk space but

cause the writing of files to be slower.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 11

WINDOWS

Window Commands

You may have up to 5 windows open at any one time. Note that when

you assemble, only one window may be open and that is to contain

the file to be assembled.

All window commands are accessed with the Alt key. When this key

is held down, the help line at the bottom of the display will

show you the available key combinations.

Add a window                                         Alt-F3

Opens another text window. You will be prompted for a file

to edit; if none is specified, NONAME.ASM will be created.

You may later save it as a named file. If too many windows

are open or if the active window is too small to divide in

half (to make room for the new window), you will get an

error message.

Close a window                                        Alt-X

Closes the current window. If there is only one window

active, this will close the window and leave IASM12.

Resize current window                                Alt-F4

Changes the size of the current window. You may adjust the

size by pressing the up and down arrow keys. When you are

done, press Enter or Esc to return to the editor.

Next window                                          Alt-F1

Makes the next text window the current window.

Previous window                                      Alt-F2

Makes the previous text window the current window.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 12

CURSOR COMMANDS

Cursor Commands

There are typically two ways of moving the cursor on screen. One

way is with the cursor control keys on the cursor pad, and the

other is via control characters. Either method can be defined or

changed by running the installation program. The symbol ^  means

hold down the Ctrl key while pressing the following letter.

Character left                             left arrow or ^S

Moves the cursor one character to the left.

Character right                           Right arrow or ^D

Moves the cursor one character to the right. This command

may cause the display to scroll horizontally but it will

not cause the cursor to change lines.

Word left                                 ^Left arrow or ^A

Moves the cursor to the beginning of the word to the left.

The cursor may move across a line break.

Word right                               ^Right arrow or ^F

Moves the cursor to the beginning of the word to the right.

The cursor may move across a line break.

Line up                                      Up arrow or ^E

Moves the cursor up a line and may scroll the screen.

Line down                                  Down arrow or ^X

Moves the cursor down a line and may scroll the screen.

Scroll up                                                ^W

Scrolls the screen up one line. The cursor does not change

lines until it hits the bottom of the screen.

Scroll down                                              ^Z

Scrolls the screen down one line. The cursor does not

change lines until it hits the top of the screen.

Page up                                          PgUp or ^R

Moves the cursor up one page with a single line of overlap.

Page down                                        PgDn or ^C

Moves the cursor down one page with a single line

of overlap.

Beginning of file                              ^PgUp or ^QR

Moves the cursor to the file's first character.

End of file                                    ^PgDn or ^QC

Moves the cursor just beyond the end of the file.

Beginning of line                               Home or ^QS

Moves the cursor to column 1 of the current line.

End of line                                      End or ^QD

Moves the cursor to the end of the current line. Trailing

blanks are always removed from lines.

Top of screen                                  ^Home or ^QE

Moves the cursor to the top line on the screen.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 13

Bottom of screen                                ^End or ^QX

Moves the cursor to the last line on the screen.

Go to line                                              ^JL

Prompts for a line number and moves the cursor to the

specified line. Any integer from 1 to 32,767 is valid. If

the value is preceded by a plus (+) or a minus (-), the

target line number will be an offset from the current line.

Go to column                                            ^JC

Prompts for a column number and moves the cursor to the

specified column of the current line. Any positive integer

from 1 to 120 is valid. If the value is preceded by a plus

(+) or a minus (-), the target column will be an offset

from the current column.

Top of block                                            ^QB

Moves the cursor to the position of the block-begin marker

set with ^KB. Works even if there is no block-end marker or

the block is hidden.

Bottom of block                                         ^QK

Moves the cursor to the position of the block-end marker

set with ^KK. Works even if there is no block-begin marker

or the block is hidden.

Previous cursor position                                ^QP

Moves to the last cursor position. Very useful after a

Save, Find or Find-and-Replace command.

Jump to marker 0..9                                ^Q0..^Q9

Moves the cursor to one of four previously set invisible

markers.

Set marker 0..9                                    ^K0..^K9

Sets one of four invisible markers at the current cursor

position.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 14

INSERT AND DELETE

Insert And Delete Commands

These commands let you insert and delete characters, words, and

lines.

New line                                        Enter or ^M

In insert mode a line break is created at the cursor

position. If autoindent is on, the cursor moves to the next

line and to the same column as the first non blank

character in the previous line, otherwise it moves to

column 1. In overwrite mode, the cursor is moved to

column 1 of the next line without inserting a new line.

Insert line                                              ^N

Inserts a line break at the cursor position. The cursor

does not change position.

Insert control character                                 ^P

This allows control characters to be entered into the text.

For example, ^P followed by ^ G will insert a ^ G (the bell

character) into the text stream. Control characters are

displayed as uppercase highlighted letters. Note that the

assembler does not accept control characters, only

the editor.

Tab                                               Tab or ^I

Moves the cursor right to the next tab stop. In insert

mode, any text to the right of the cursor on the current

line is also moved. In overwrite mode, only the cursor

moves. The amount of cursor movement depends on whether

fixed tabs or smart tabs are enabled. See the section

on tabs.

Delete current character                          Del or ^G

Deletes the character at the cursor position. Characters to

the right on the same line are moved one position left. The

command does not cross line breaks.

Delete character left                            Bksp or ^H

Moves the cursor to the left one and deletes the character

at that position. Characters on the same line are moved one

position left. If the cursor starts in column one, the

current line is joined to the line above it.

Delete word                                              ^T

Deletes the word to the right of the cursor. This command

works across line breaks and may be used to remove line

breaks.

Delete to end of line                                   ^QY

Deletes all characters from the current cursor position to

the end of the line.

Delete line                                              ^Y

Deletes the line containing the cursor and moves any lines

below it up one line. The cursor moves to column one of the

next line. A line deleted this way cannot be restored.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 15

BLOCK COMMANDS

Block Commands

A block is any defined, contiguous stream of text. It may be a

single character or many lines. In fact, the entire file can be

specified to be a block. A block is defined by placing a

begin-block marker at the first character in the desired block

and an end-block marker after the last character. Once a block is

so marked, it may be moved, copied, deleted or written to a file.

Once a block is defined, it is typically shown highlighted. This

may be changed with the hide-block command. All of the block

manipulation commands work only with a non-hidden fully defined

block.

Begin block                                             ^KB

This is the marker for the beginning of a block. The marker

itself is not visible on the screen. If an end-block

command has been given, the block will now be highlighted.

Whether or not an end-block exists, you can jump to the

begin-block marker at any time with the Top of block

command, ^QB.

End block                                         ^KK or F8

This is the marker for the end of a block. The marker

itself is not visible on the screen. If a begin-block

command has been given, the block will now be highlighted.

Whether or not an begin-block exists, you can jump to the

end-block marker at any time with the Bottom of block

command, ^QK.

Copy block                                              ^KC

A copy of the previously marked and displayed block is

placed at the current cursor position. The original block

is not effected and the markers are moved to the new copy.

Move block                                              ^KV

The previously marked and displayed block is moved to the

current cursor position. The block markers remain around

the block as it is moved.

Delete block                                            ^KY

Deletes a previously marked and displayed block. Although

the Undo last deletion command (^QU) can usually restore

portions of an accidentally deleted block, there is no

command to restore a deleted block in its entirety, so use

this command with care.

Hide block                                              ^KH

Turns on and off the highlighting of a block. The block

commands only work on displayed blocks. This command does

not effect the markers.

Print block                                             ^KP

Prints the block currently selected. Make sure the printer

is connected and turned on. The command is canceled

with Esc.

Mark single word                                        ^KT

Marks an entire word as a block (places the begin-block and

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 16

end-block markers around the word) in one command. If the

cursor is within a word, that word is marked. If the cursor

is not within a word, then the word to the left is marked.

Read block from file                                    ^KR

Reads an entire file into the text stream at the current

cursor position as if it were a marked block within the

text. The file is read in and marked as a block. The editor

will prompt you for the name of the file. If the command

has previously been used, the last file name requested will

be displayed. You may either enter a new name, change the

one displayed by using the Bksp key, or accept it by

pressing Enter. The file name specified may be any legal

file name, including a drive and/or path identifier. The

command is canceled with Esc.

Write block to file                                     ^KW

Copies the previously marked and displayed block to a file.

The block itself is not changed and the markers are left in

place. The editor will prompt you for the name of the file.

You should not use names with the extension .BAK since this

is the extension used when the editor creates a backup

file. If the file name specified is found to exist, you are

asked if you want to overwrite it. If you type N (for No),

you can enter a new file name, change the one displayed by

using the Bksp key, or cancel the command using Esc. If no

block has been specified, this command is ignored.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 17

MISCELLANEOUS

Miscellaneous Commands

Abort                                                   Esc

Halts an operation in progress. The keyboard buffer is

checked regularly to see if the Abort command has been

issued; if it has, the buffer is emptied and the operation

is stopped.

Toggle insert mode                                Ins or ^V

Selects either insert or overwrite mode. In insert mode,

the text to the right of the cursor is moved to the right

as new text is entered. In overwrite mode, the new text

replaces any old text. The status line says "Insert" when

in insert mode.

Toggle autoindent                                       ^QI

When autoindent is enabled, the cursor will go to the next

row and to the same column as the first non-blank character

on the previous line in response to a ^M or Enter. The

status line says "Indent" when this is enabled.

Toggle fixed tabs                                       ^QT

When fixed tabs are enabled, the first tab stop is at

column 9 and then there is one every 8 columns thereafter.

The default is not fixed tabs but rather smart tabs. Smart

tabs are located at the first character of each word on the

previous line. This makes for very clean looking source

code. When fixed tabs are enabled, the status line says

"Tab" and the editor uses spaces in the text stream. Smart

tabs are also implemented with a series of blanks.

Restore line                                            ^QL

This command will undo any changes to the current line as

long as the cursor has not left the line. The line will be

restored regardless of any changes made to it.

Undo last deletion                                      ^QU

Restores lines deleted with the Delete line command or the

Delete command. It does not restore single characters or

words. To undo your most recent changes to the current

line, use the Restore line command. The size of the "undo

buffer" (used to save deleted lines) is specified with the

Set undo limit command.

Set undo limit                                          ^JU

Sets the size of the "undo buffer" which stores deleted

lines. The default value is 40 lines unless you have

changed it with the installation program.

Show version                                            ^JV

Display the current version of IASM12.

Show available memory                                   ^JR

Shows the amount of RAM available to IASM12.

Find                                                    ^QF

Find lets you search for a character string up to 67

characters in length. The status line will clear and you

will be prompted for a search string. If the command has

been used previously, the last search string will be

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 18

displayed. You can select it again by pressing Enter,

edit it, or enter an entire new string. When editing a

string you may use Bksp, ^ R (restores previous string),

^S (move cursor left), and ^D (move cursor right). Esc

will cancel the command. To enter a control character

into the search string, precede it with ^P (for example,

to find a period '.' at the end of a line you would enter

the search string : . ^P ^M ). After the search string is

entered you may specify search options. If any options

were used in a previous search, they are offered and may

be edited. The following options are available:

B   Searches from the current cursor position towards

the beginning of the file (Backwards).

G   Searches globally. The entire file is scanned for

the search string, regardless of the current

position of the cursor. The search starts at the

beginning of the file if searching forwards; at the

end if searching backwards.

L   Limits the search to the currently marked block.

U   Ignore case; treats all alphabetic characters as if

they were uppercase.

The cursor is placed at the end of the matching pattern

if one is found.

Find and Replace                                        ^QA

This command works almost exactly as the previous one. The

difference is that after the search string is prompted for,

you are asked for the replacement string. This new string

may be up to 67 characters long. The previous replacement

string (if there is one) will be displayed and it may be

accepted, edited, or replaced. Finally, you are prompted

for options.

Any previously used options are shown and may be edited as

before. The same options are available as are for the Find

command with the addition of :

N Replaces without a prompt. Normally the user is prompted

as to whether or not to make the change (No prompt).

If a match with the search string is found in the text, the

cursor is placed at the end of the target. You are then

asked if you want the replacement to occur. Press Y to

replace, or N to skip, or A to replace it and all

subsequent matches without asking. The operation can be

aborted by hitting Q or Esc. If the N option was selected,

this question will not be asked. You can speed up a global

search and replace operation by selecting the N option and

then pressing Esc. The screen will not be updated until

all updates are completed.

Find next                                                ^L

This command will repeat the last search operation, either

a Find or Find and Replace.

Exit editor                                             ^KD

Saves the current file and exits the editor. The original

file (if one exists) is renamed FILENAME.BAK.

Exit editor                                           Alt-X

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 19

If the file in the editor has been modified, a prompt asks

if the file should be saved. If yes, the file is saved and

IASM12 exits to DOS, else IASM12 simply exits.

Save to file                                            ^KN

Prompts you for a file name, then saves the file in the

current window to the specified name. This becomes the new

file in the current window. This is especially useful for

NONAME.ASM files.

==================================================================

ASSEMBLER

Assembler

The assembler takes the file currently in the editor and

assembles it. Depending upon the options chosen, the assembler

may produce an object file, map file, and/or a listing file. The

source file uses factory standard mnemonics. The list of

acceptable mnemonics may be seen using the environment's help

screens.

Statements - Each line of the source file contains a single

assembly language statement. A statement may contain up to four

fields: label, operation, operands, and comment fields, as

follows :

label   operation operand       ;comment

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 20

LABELS

Labels

Labels, if present, must start in column one. They may be up to

16 characters long and must start with an alphabetic character.

The assembler does not differentiate between upper and lower case

letters when searching for labels. The label itself may contain

any alphanumeric character, underscore, or dash. An optional

colon may be added at the end of a label but a space will

suffice.

Note that labels within macros must be no longer than 10

characters.

Examples:

Label: ThisIsALabel: Loop_1 This_label_is_much_too_long:

The label in the last example would be truncated to 16

characters and, to the assembler, would be the same as:

This_label_is_much_longer_than_needed

==================================================================

DIRECTIVES

Assembler Directives

These are keywords which control the progress and the modes of

the assembler itself. They are invoked by placing a /, #, or $ as

the first character of a line followed by the following directive

words and parameters:

BASE ^          Change default input base to ^

(2,8,10, or 16 decimal).

CYCLE_ADDER_ON  Start accumulating instruction cycles.

CYCLE_ADDER_OFF Stop accumulating instruction cycles and

print total.

ELSEIF          Alternate code for conditional assembly.

ENDIF           End of conditional assembly.

IF ^            Conditional assembly if condition

"^" is true.

IFNOT ^         Conditional assembly if condition "^"

is false.

INCLUDE '^ '    Include file named "^ " in source code.

MACRO ^         MacroCreates a macro with the name "^ ".

MACROEND        Ends a macro definition.

SET ^           Set condition named "^ " to true.

SETNOT ^        Set condition named "^ " to false.

Each directive is fully explained on the following pages.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 21

BASE

Base

The BASE assembler directive changes the default base of the

current file. The parameter specified must be in the current base

or have a base qualifier on it. The new base will be in effect

until either the end of the file or until another BASE directive

is encountered. The default base is hexadecimal. You may set the

base to either 2, 8, 10 or 16 decimal.  It is a good idea to

specify an explicit base (see the section on constants) since you

may not be sure what base is currently in use.

Examples:

$BASE 10T       ;changes default base to decimal

$BASE 0AH       ;changes default base to decimal

==================================================================

CYCLE_ADDER

Cycle_adder

The CYCLE_ADDER_ON and CYCLE_ADDER_OFF assembler directives

control a counter that is internal to the assembler. When the

CYCLE_ADDER_ON directive is encountered, this internal counter is

cleared. It is then incremented by the number of instruction

cycles associated with each instruction subsequently assembled.

When the CYCLE_ADDER_OFF directive is reached, the counter is

disabled and the total number of cycles counted is displayed in

the .LST file. Some instructions have various cycle counts,

depending on the actual execution of the instruction (test and

branch, etc.). The smaller number of cycles will be used by IASM12.

NOTE: The Listing and Cycle Cntr options in the Assemble submenu

must be on for this directive to work.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 22

CONDITIONALS

Conditional Assembly

Conditional assembly allows you to specify blocks of code that

should assemble only on certain conditions. These conditions are

specified with the SET and SETNOT directives.

The SET directive sets the parameter to a "true" value. The

SETNOT directive sets it to "false". Any code following an IF

directive (up to either an ENDIF or an ELSEIF) will be assembled

if the given parameter has been set true. Any code following an

IFNOT directive (up to either an ENDIF or an ELSEIF) will be

assembled if the given parameter has been set false.

An ELSEIF directive may come between an IF or IFNOT directive and

an ENDIF. The code between the ELSEIF and the ENDIF will be

assembled only if the code between the IF (or IFNOT) and ELSEIF

was not assembled. Likewise, if the code between the IF (or

IFNOT) and the ELSEIF was not assembled, the code between the

ELSEIF and ENDIF will be.

An example is definitely called for ....

EXAMPLE:

$SET    debug     ;set debug = true

$SETNOT test      ;set test = false

nop       ;always assembles

nop       ;always assembles

$IF     debug     ;if debug = true

jmp start ;will assemble

$ELSEIF           ;if debug = false

jmp end   ;will not assemble

$ENDIF            ;

nop       ;always assembles

nop       ;always assembles

$IF test          ;if test = true

jmp test  ;will not assemble

$ENDIF            ;

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 23

INCLUDE

Include

The INCLUDE directive tells the assembler where to look for

source code. Once an include directive is encountered the

assembler starts taking source code from the specified file

until either another INCLUDE is encountered or the end of file is

found. If the end of file is found, the assembler continues with

the source code immediately following the INCLUDE directive.

Includes may be nested up to a maximum depth of ten. The file

specified should contain a full path name if it does not reside

in the current directory.

You can choose whether or not to show the source code from

INCLUDE files within the .LST file. This is controlled via a

choice in the Assemble submenu.

Note that the file name must be in quotes (single or double).

EXAMPLES:

$INCLUDE 'init.asm'

$INCLUDE "c:\project\init.asm"

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 24

MACROS

Macros

The basic idea of a macro is simple. You assign a name to a

block of text and whenever that name is encountered during

assembly, the block of text is inserted. The macro can be

thought of as expanding into the text when it is referred to.

A macro is very similar to an INCLUDE file but it is more

flexible. If the same file is INCLUDEd more than once, any

labels in it will cause errors. A macro, on the other hand, may

have labels and may also be passed parameters.

Macros are defined with the MACRO directive. All source code

between a MACRO and MACROEND directive is part of the macro. No

directives are allowed within a macro. The macro definition

itself does not contain any parameters. Within the macro, any

place a parameter is needed, the parameter is specified with %n

where n is the nth parameter at the macro call. If parameters are

specified on the macro definition line, they are ignored, hence

they may be helpful for documentation.

Note that macros cannot be forward referenced.

Example 1:

$MACRO divide_by_4

asr a   ; acc / 2

asr a   ; acc /4

$MACROEND

In this example, the accumulator is divided by four.

Example 2:

$MACRO delay count

mov a,%1

loop:   nop

djnz loop

$MACROEND

This example creates a time delay. The parameter count does

nothing except help with documentation showing that the first

parameter is a count. The parameter sent to the routine will be

substituted for the %1 wherever it occurs. The first time this

macro is called, the label loop will be changed by the assembler

to loop:0001 as will the operand on the last line. If this macro

is invoked with :

delay 100t then the loop will occur 100 (base ten)

times.

Extra parameters sent to a macro will be ignored but an error

will occur if not enough are sent.

Labels will automatically be changed each time they are used.

Labels within macros cannot be longer than 10 characters. The

assembler will append :nnnn to the label where nnnn is a four

digit hexadecimal number. This will ensure that labels are always

unique. Note that you may not have any code jump into a macro but

you may jump out of a macro.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 25

You have the choice as to whether the code generated during a

macro is printed in the listing file or not. The setting in the

ASSEMBLE sub-menu handles this. If the setting is "MACROS hide"

then you will only see the macro call. If the setting is "MACRO

view" then the code will be shown. The code shown will not look

exactly like that entered into the macro definition. No comments

will appear and the code will be all upper case. Note that these

settings do not affect the macro definition itself, it always

appears in the listing file.

==================================================================

CONSTANTS

Constants

Constants are specific numbers entered into assembly language

instructions. The default base for all constants in the assembler

is hexadecimal. The default may be changed with the base

assembler directive. Temporary override of the default is

possible as follows:

Base    Prefix  Suffix

2       %       Q

10      !       T

16      $       H

Either a prefix or a suffix is used, not both. Additionally, a $

or a * by itself is used to mean "the current program counter".

Examples : 10010111Q = %10010111 = 97H = $97

JMP $   ; jump to myself

JMP *   ; jump to myself

An ASCII constant may be specified by placing it in single or

double quotes such as 'A' which is the same as 41H. Strings

consist of characters enclosed in quotes as in :

db 'this is a string'

==================================================================

OPCODES

Opcodes

All factory mnemonics are supported. The full list of supported

mnemonics for the assembler can be seen in the on-line help

system by looking under "INSTRUCTION SET". Opcodes cannot start

in column one and there must be at least one space or a colon

between the label (if one exists on the line) and the opcode.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 26

OPERANDS

Operands

An operand may be an address, label, or a constant as defined by

the opcode. Arithmetic may be performed within parameters, at

assembly time. The allowed operators are:

*       multiplication

/       division

\       special division

+       addition

-       subtraction

<       left shift

>       right shift

%       remainder after division

&       bitwise and

|       bitwise or

^       bitwise xor

Special division is an operator that allows a divide by zero. If

there is a divide by zero anywhere in the expression, the entire

expression equates to zero.

Operator precedence follows the rules of algebra and may be

altered by the use of parentheses.

If more then one operator, parenthesis, or imbedded spaces are

used, the entire expression must be put inside curly brackets {}.

Examples:

jmp start       ;start is a previously defined label.

jmp start+3     ;jump to location start + 3

jmp {start > 2} ;jump to location start divided by 4

==================================================================

COMMENTS

Comments

Comments are delineated by a semicolon. They may start in any

column and go until the end of the line. Additionally, any line

with an * or a ; in column 1 is considered to be a comment.

Examples:

nop     ;this is a comment

* this entire line is a comment.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 27

PSEUDO-OPS

Pseudo-Ops

The following list of pseudo operations are allowed in place of

opcodes:

ds n or

rmb n     Define storage, n = number or label,

reserves n bytes. Forward references of n

are not allowed.

db m or

fcb m     Define byte storage, m = label, number or

string. Strings generate ASCII codes for

multiple bytes. Numbers and labels define

only a single byte per parameter. Multiple

parameters are allowed and are separated

by commas.

dw  n or

fdb n     Define word storage, n=label, number or string.

Two bytes are generated for each number or

label.  Multiple parameters are allowed and are

separated by commas.

lab: equ n  Assigns the value of the number or label n to

the label lab. Forward references to n are

not allowed.

org n     Sets the origin to the value of the number or

label n. Forward references to n are not

allowed.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 28

LISTING DIRECTIVES

Listing Directives

These are keywords entered into the source code that control the

output to the listing file. They are purely for viewing the

source code output and have no effect on the actual code

assembled.

A listing directive is invoked by placing a period '.' in column

one of the source code immediately followed by the directive.

Note that the directive itself will not appear in the listing

file. If you want it to, then invoke it as you would any

assembler directive, with a /, #, or $.

eject or

page        Begins a new page.

header '^'  The string specified will be a header on the

listing pages. The string must be in quotes

and can only be defined once.

Default : header ' '

list        Turns on the .lst file output if the list

choice in the assemble sub-menu is on and

the .lst file output has been turned off with

a nolist directive. The list choice in the

assemble sub-menu MUST be on for this optional

directive to work.

nolist      Turns off the .lst file output if it has been

turned on. This directive effects the listing

of the symbol table.

pagelength ^  Sets the length of the page in lines.

Default: pagelength 66t

pagewidth ^  Sets the width of the output. Any additional

text will be wordwrapped.

Default : pagewidth 80t

subheader '^'  The string specified will be a subheader on

the listing pages. The string must be in

quotes and will take effect on the

next page.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 29

LISTING FILE

Listing File

When a listing file is requested via the menu system, it is

created during assembly. The listing file is given the same name

as the file being assembled with the extension .LST. Note that

any existing file with this name will be overwritten. The listing

file has the following format :

AAAA VVVVVVVV [CC] LLLL Source Code .....

The first four hexadecimal digits (AAAA) are the address of the

instruction in the target processor's memory.

The next group of hexadecimal digits (VVVVVVVV) are the values

put into that memory address and possibly the next several

addresses. The size of this field depends on the actual opcode.

If Cycle Cntr was turned on prior to assembly, the number of

machine cycles (CC) used by the opcode is displayed in decimal

within square brackets. In instances where an instruction has

several possible cycle counts (whether or not a branch happens,

for instance) and the assembler cannot determine the correct

number, the best case (lowest number) is given.

The next decimal digits (LLLL _ up to four places) comprise the

line count.

Finally, the actual source code is presented.

The end of the listing file contains the symbol table with every

label used and it's value presented.

This file is affected by the listing directives list and nolist.

A nolist directive at the end of the source code file will

suppress the symbol table.

==================================================================

OBJECT FILES

Object files:

When an object file is requested via the menu system, it is

created during assembly. The object file is given the same name

as the file being assembled with the extension .HEX or .S19. The

choice of name depends on the choice made in the Assemble

sub-menu. Note that any existing file with the same name will be

overwritten.

HEX is Intel 8-bit object code format. S19 is Motorola 8-bit

object code format.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 30

MAP FILES

Map files:

When requested via the menu system, map files are generated

during assembly. The file is used by other P&E Microcomputer

products to aid in symbolic debugging.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 31

ERRORS

Errors

If the assembler finds an error during assembly, the error is

highlighted and an error message appears on the prompt line. The

possible errors and their meanings follow.

error 1  Parameter : invalid, too large, missing, or

out of range.

There is an error in the operand field of the

highlighted line. It may be an invalid number

or representation of a number, or the parameter

evaluates to a number that does not fit in the

memory space allocated to the instruction.

error 2  Too many conditional assembly variables.

You may have up to a maximum of 25 conditional

variables.

error 3  Conditional assembly variable not found.

The variable in the IF or IFNOT statement has not

been declared with a SET of SETNOT.

error 4  Include directives nested too deep.

Includes may only be nested 10 levels.

error 5  Invalid base value.

The base parameter may be set to 2, 8, 10 or 16

decimal.  Since you may not know the current

base, it is always a good idea to use a value

with an explicit base identifier such as 10T

or 0AH.

error 6  Undefined label.

The label used as a parameter in the highlighted

line has not been declared.

error 7  Too many labels.

See error 16.

error 8  MACRO parameter error.

The macro is expecting a different number of

parameters than it received.

error 9  MACRO label too long.

Macro labels may be up to 10 characters, no more.

error 10 INCLUDE file not found.

The file specified in the INCLUDE directive was

not found. Specify the full path name if

necessary. There is no default extension, it must

be specified if it exists.  Be sure that the file

name is enclosed in quotes.

error 11 Unrecognized operation.

The highlighted opcode is unknown. This may be

caused by a wrong number or type of parameters.

error 12 Duplicate label.

The label on the highlighted line has already

been used.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 32

error 13 Invalid opcode, too long.

The opcode on the highlighted line is wrong.

error 14 Error writing .LST or .MAP file

- check disk space.

For some reason a .lst or .map file cannot be

created. Check disk space and make sure that

your CONFIG.SYS file allows multiple files to

be open at once (see your DOS manual).

error 15 Error writing object file - check disk space.

See error 14.

error 16 Out of memory.

The assembler ran out of system memory. Try

creating a file that consists of an INCLUDE

directive only.  This should specify your primary

file. Assembling this file will leave the maximum

amount of memory available to the assembler.

error 17 '}' not found.

A mathematical expression that started with '{'

did not end with '}'.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 33

COMMUNICATIONS

Communications

Comm Hitting F7 opens the communications window. The parameters

used (port, baud, parity, word length, number of stop bits) are

based on the options in the COMM sub-menu. If you are unsure of

what these settings should be, consult the manual that came with

the development board in use. Once the communications window is

opened, you can toggle between communicating and editing with the

F7 key. Note that if you change the communication parameters

while the communication window is open, the parameters will not

take effect until the window is closed and then reopened. The

speed of the download command is determined by the baud rate

chosen and the inter character delays set in the installation

program. Once inside the communications window, the following hot

keys are available :

F1    : Help     This key will bring up the help system.

F2    : View     This key will cause a prompt for a file

name. Entering a valid file name will

cause that file to be loaded into memory

for viewing. The file may be up to the

size of available memory. When the

communications window is left, the viewer

will close and be replaced by the editor.

F6    : Download Hitting F6 causes a prompt for a file

name. Entering a valid file name will

cause that file to be transferred out

the appropriate serial port. No special

protocol is used, the file is simply

transferred as is.

F7    : Edit     Hitting F7 will put you into the editor

window, closing an open view file.

F8,F9 : Resize   By using F8 and F9 the communications

window is made larger and smaller.

F10   : Close    Hitting F10 will close the communications

window and return control, and the full

screen, back to the editor.

==================================================================

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 34

Some technical data in the help system is used by permission of

Motorola Semiconductor.

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 35

INSTRUCTION SET

Instruction Set

ABA     add accumulator B to accumulator A

ABX     add accumulator B to index reg. X

ABY     add accumulator B to index reg. Y

ADCA    add with carry to A

ADCB    add with carry to B

ADDA    add without carry to A

ADDB    add without carry to B

ADDD    add double accumulator

ANDA    logical and A

ANDB    logical and B

ANDCC   logical and CCR with mask

ASL     arithmetic shift left memory

ASLA    arithmetic shift left A

ASLB    arithmetic shift left B

ASLD    arithmetic shift left double acc.

ASR     arithmetic shift right memory

ASRA    arithmetic shift right A

ASRB    arithmetic shift right B

BCC     branch if carry clear

BCLR    clear bit(s) in memory

BCS     branch if carry set

BEQ     branch if equal

BGE     branch if >= zero

BGND    enter background debug mode

BGT     branch if > zero

BHI     branch if higher

BHS     branch if higher or same

BITA    bit test A

BITB    bit test B

BLE     branch if <= zero

BLO     branch if lower

BLS     branch if lower or same

BLT     branch if < zero

BMI     branch if minus

BNE     branch if not equal to zero

BPL     branch if plus

BRA     branch always

BRCLR   branch if bit(s) clear

BRN     branch never

BRSET   branch if bit(s) set

BSET    set bit(s) in memory

BSR     branch to subroutine

BVC     branch if overflow clear

BVS     branch if overflow set

CALL    call subroutine in extended memory

CBA     compare accumulators

CLC     clear carry

CLI     clear interrupt mask

CLR     clear memory

CLRA    clear A

CLRB    clear B

CLV     clear two's complement overflow bit

CMPA    compare A

CMPB    compare B

COM     complement memory

COMA    complement A

COMB    complement B

CPD     compare accumulator D

CPS     compare stack pointer

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 36

CPX     compare index reg. X

CPY     compare index reg. Y

DAA     decimal adjust A

DBEQ    decrement and branch if equal to zero

DBNE    decrement and branch if not equal to zero

DEC     decrement memory

DECA    decrement A

DECB    decrement B

DES     decrement stack pointer

DEX     decrement index register X

DEY     decrement index register Y

EDIV    extended divide 32-bit by 16-bit (unsigned)

EDIVS   extended divide 32-bit by 16-bit (signed)

EMACS   extended multiply and accumulate (signed)

EMAXD   max of 2 unsigned 16-bit values (result in D)

EMAXM   max of 2 unsigned 16-bit values (result in mem)

EMIND   min of 2 unsigned 16-bit values (result in D)

EMINM   min of 2 unsigned 16-bit values (result in mem)

EMUL    extended multiply 16-bit by 16-bit (unsigned)

EMULS   extended multiply 16-bit by 16-bit (signed)

EORA    exclusive or A

EORB    exclusive or B

ETBL    extended table lookup and interpolate

EXG     exchange register contents

FDIV    fractional divide

IBEQ    increment and branch if equal to zero

IBNE    increment and branch if not equal to zero

IDIV    integer divide

IDIVS   integer divide (signed)

INC     increment memory

INCA    increment A

INCB    increment B

INS     increment stack pointer

INX     increment index register X

INY     increment index register Y

JMP     jump

JSR     jump to subroutine

LBCC    long branch if carry clear

LBCS    long branch if carry set

LBEQ    long branch if equal

LBGE    long branch if greater than or equal to zero

LBGT    long branch if greater than zero

LBHI    long branch if higher

LBHS    long branch if higher or same

LBLE    long branch if less than or equal to zero

LBLO    long branch if lower

LBLS    long branch if lower or same

LBLT    long branch if less than zero

LBMI    long branch if minus

LBNE    long branch if not equal to zero

LBPL    long branch if plus

LBRA    long branch always

LBRN    long branch never

LBVC    long branch if overflow clear

LBVS    long branch if overflow set

LDAA    load accumulator A

LDAB    load accumulator B

LDD     load double accumulator

LDS     load stack pointer

LDX     load index register X

LDY     load index register Y

LEAS    load stack pointer with effective address

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 37

LEAX    load X with effective address

LEAY    load Y with effective address

LSL     logical shift left memory

LSLA    logical shift left A

LSLB    logocal shift left B

LSLD    logical shift left double

LSR     logical shift right memory

LSRA    logical shift right A

LSRB    logical shift right B

LSRD    logical shift right double accumulator

MAXA    max of 2 unsigned 8-bit values (result in A)

MAXM    max of 2 unsigned 8-bit values (result in mem)

MEM     determine grade of membership

MINA    min of 2 unsigned 8-bit values (result in A)

MINM    min of 2 unsigned 8-bit values (result in mem)

MOVB    move data from one memory byte to another

MOVW    move data from one memory word to another

MUL     multiply unsigned

NEG     negate memory

NEGA    negate A

NEGB    negate B

NOP     no operation

ORAA    inclusive or A

ORAB    inclusive or B

ORCC    logical or CCR with mask

PSHA    push A onto stack

PSHB    push B onto stack

PSHC    push CCR onto stack

PSHD    push double accumulator onto stack

PSHX    push index reg. X onto stack

PSHY    push index reg. Y onto stack

PULA    pull A from stack

PULB    pull B from stack

PULC    pull CCR from stack

PULD    pull double accumulator from stack

PULX    pull index reg. X from stack

PULY    pull index reg. Y from stack

REV     fuzzy logic rule evaluation

REVW    fuzzy logic rule evaluation (weighted)

ROL     rotate left memory

ROLA    rotate left A

ROLB    rotate left B

ROR     rotate right memory

RORA    rotate right A

RORB    rotate right B

RTC     return from call

RTI     return from interrupt

RTS     return from subroutine

SBA     subtract accumulators

SBCA    subtract with carry from A

SBCB    subtract with carry from B

SEC     set carry

SEI     set interrupt mask

SEV     set two's complement overflow bit

SEX     sign extend into 16-bit register

STAA    store accumulator A

STAB    store accumulator B

STD     store double accumulator

STOP    stop processing

STS     store stack pointer

STX     store index register X

STY     store index register Y

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 38

SUBA    subtract A

SUBB    subtract B

SUBD    subtract double accumulator

SWI     software interrupt

TAB     transfer from acc. A to acc. B

TAP     transfer from acc. A to CCR

TBA     transfer from acc. B to acc. A

TBEQ    test and branch if equal to zero

TBL     table lookup and interpolate

TBNE    test and branch if not equal to zero

TFR     transfer register content to another register

TPA     transfer from CCR to accumulator A

TRAP    unimplemented opcode trap

TST     test memory

TSTA    test A

TSTB    test B

TSX     transfer from SP to index reg. X

TSY     transfer from SP to index reg. Y

TXS     transfer from index reg. X to SP

TYS     transfer from index reg. Y to SP

WAI     wait for interrupts

WAV     weighted average

XGDX    exchange double acc. and index reg. X

XGDY    exchange double acc. and index reg. Y

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 39

INSTRUCTION FORMATS

Note that the symbol ^ reflects a numeric parameter and the

symbol ! represents an addressing mode..

ABA            ABX            ABY            ADCA !,!

ADCA #^        ADCA ,!        ADCA @^        ADCA [!,!]

ADCA [^,!]     ADCA ^         ADCA ^,!       ADCB !,!

ADCB #^        ADCB ,!        ADCB @^        ADCB [!,!]

ADCB [^,!]     ADCB ^         ADCB ^,!       ADDA !,!

ADDA #^        ADDA ,!        ADDA @^        ADDA [!,!]

ADDA [^,!]     ADDA ^         ADDA ^,!       ADDB !,!

ADDB #^        ADDB ,!        ADDB @^        ADDB [!,!]

ADDB [^,!]     ADDB ^         ADDB ^,!       ADDD !,!

ADDD #^        ADDD ,!        ADDD @^        ADDD [!,!]

ADDD [^,!]     ADDD ^         ADDD ^,!       ANDA !,!

ANDA #^        ANDA ,!        ANDA @^        ANDA [!,!]

ANDA [^,!]     ANDA ^         ANDA ^,!       ANDB !,!

ANDB #^        ANDB ,!        ANDB @^        ANDB [!,!]

ANDB [^,!]     ANDB ^         ANDB ^,!       ANDCC #^

ASL !,!        ASL ,!         ASL [!,!]      ASL [^,!]

ASL ^          ASL ^,!        ASLA           ASLB

ASLD           ASR !,!        ASR ,!         ASR [!,!]

ASR [^,!]      ASR ^          ASR ^,!        ASRA

ASRB           BCC ^          BCLR !,!,^     BCLR ,!,^

BCLR @^,^      BCLR ^,!,^     BCLR ^,^       BCS ^

BEQ ^          BGE ^          BGND           BGT ^

BHI ^          BHS ^          BITA !,!       BITA #^

BITA ,!        BITA @^        BITA [!,!]     BITA [^,!]

BITA ^         BITA ^,!       BITB !,!       BITB #^

BITB ,!        BITB @^        BITB [!,!]     BITB [^,!]

BITB ^         BITB ^,!       BLE ^          BLO ^

BLS ^          BLT ^          BMI ^          BNE ^

BPL ^          BRA ^          BRCLR !,!,^,^  BRCLR ,!,^,^

BRCLR @^,^,^   BRCLR ^,!,^,^  BRCLR ^,^,^    BRN ^

BRSET !,!,^,^  BRSET ,!,^,^   BRSET @^,^,^   BRSET ^,!,^,^

BRSET ^,^,^    BSET !,!,^     BSET ,!,^      BSET @^,^

BSET ^,!,^     BSET ^,^       BSR ^          BVC ^

BVS ^          CALL !,!,^     CALL ,!,^      CALL [!,!]

CALL [^,!]     CALL ^,!,^     CALL ^,^       CBA

CLC            CLI            CLR !,!        CLR ,!

CLR [!,!]      CLR [^,!]      CLR ^          CLR ^,!

CLRA           CLRB           CLV            CMPA !,!

CMPA #^        CMPA ,!        CMPA @^        CMPA [!,!]

CMPA [^,!]     CMPA ^         CMPA ^,!       CMPB !,!

CMPB #^        CMPB ,!        CMPB @^        CMPB [!,!]

CMPB [^,!]     CMPB ^         CMPB ^,!       COM !,!

COM ,!         COM [!,!]      COM [^,!]      COM ^

COM ^,!        COMA           COMB           CPD !,!

CPD #^         CPD ,!         CPD @^         CPD [!,!]

CPD [^,!]      CPD ^          CPD ^,!        CPS !,!

CPS #^         CPS ,!         CPS @^         CPS [!,!]

CPS [^,!]      CPS ^          CPS ^,!        CPX !,!

CPX #^         CPX ,!         CPX @^         CPX [!,!]

CPX [^,!]      CPX ^          CPX ^,!        CPY !,!

CPY #^         CPY ,!         CPY @^         CPY [!,!]

CPY [^,!]      CPY ^          CPY ^,!        DAA

DBEQ !,^       DBNE !,^       DEC !,!        DEC ,!

DEC [!,!]      DEC [^,!]      DEC ^          DEC ^,!

DECA           DECB           DES            DEX

DEY            EDIV           EDIVS          EMACS ^

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 40

EMAXD !,!      EMAXD ,!       EMAXD [!,!]    EMAXD [^,!]

EMAXD ^,!      EMAXM !,!      EMAXM ,!       EMAXM [!,!]

EMAXM [^,!]    EMAXM ^,!      EMIND !,!      EMIND ,!

EMIND [!,!]    EMIND [^,!]    EMIND ^,!      EMINM !,!

EMINM ,!       EMINM [!,!]    EMINM [^,!]    EMINM ^,!

EMUL           EMULS          EORA !,!       EORA #^

EORA ,!        EORA @^        EORA [!,!]     EORA [^,!]

EORA ^         EORA ^,!       EORB !,!       EORB #^

EORB ,!        EORB @^        EORB [!,!]     EORB [^,!]

EORB ^         EORB ^,!       ETBL !,!       ETBL ,!

ETBL ^,!       EXG !,!        FDIV           IBEQ !,^

IBNE !,^       IDIV           IDIVS          INC !,!

INC ,!         INC [!,!]      INC [^,!]      INC ^

INC ^,!        INCA           INCB           INS

INX            INY            JMP !,!        JMP ,!

JMP [!,!]      JMP [^,!]      JMP ^          JMP ^,!

JSR !,!        JSR ,!         JSR @^         JSR [!,!]

JSR [^,!]      JSR ^          JSR ^,!        LBCC ^

LBCS ^         LBEQ ^         LBGE ^         LBGT ^

LBHI ^         LBHS ^         LBLE ^         LBLO ^

LBLS ^         LBLT ^         LBMI ^         LBNE ^

LBPL ^         LBRA ^         LBRN ^         LBVC ^

LBVS ^         LDAA !,!       LDAA #^        LDAA ,!

LDAA @^        LDAA [!,!]     LDAA [^,!]     LDAA ^

LDAA ^,!       LDAB !,!       LDAB #^        LDAB ,!

LDAB @^        LDAB [!,!]     LDAB [^,!]     LDAB ^

LDAB ^,!       LDD !,!        LDD #^         LDD ,!

LDD @^         LDD [!,!]      LDD [^,!]      LDD ^

LDD ^,!        LDS !,!        LDS #^         LDS ,!

LDS @^         LDS [!,!]      LDS [^,!]      LDS ^

LDS ^,!        LDX !,!        LDX #^         LDX ,!

LDX @^         LDX [!,!]      LDX [^,!]      LDX ^

LDX ^,!        LDY !,!        LDY #^         LDY ,!

LDY @^         LDY [!,!]      LDY [^,!]      LDY ^

LDY ^,!        LEAS !,!       LEAS ,!        LEAS ^,!

LEAX !,!       LEAX ,!        LEAX ^,!       LEAY !,!

LEAY ,!        LEAY ^,!       LSL !,!        LSL ,!

LSL [!,!]      LSL [^,!]      LSL ^          LSL ^,!

LSLA           LSLB           LSLD           LSR !,!

LSR ,!         LSR [!,!]      LSR [^,!]      LSR ^

LSR ^,!        LSRA           LSRB           LSRD

MAXA !,!       MAXA ,!        MAXA [!,!]     MAXA [^,!]

MAXA ^,!       MAXM !,!       MAXM ,!        MAXM [!,!]

MAXM [^,!]     MAXM ^,!       MEM            MINA !,!

MINA ,!        MINA [!,!]     MINA [^,!]     MINA ^,!

MINM !,!       MINM ,!        MINM [!,!]     MINM [^,!]

MINM ^,!       MOVB #^,^      MOVB #^,,!     MOVB #^,^,!

MOVB #^,!,!    MOVB ^,^       MOVB ^,,!      MOVB ^,^,!

MOVB ^,!,!     MOVB ,!,^      MOVB ^,!,^     MOVB !,!,^

MOVB ,!,,!     MOVB ,!,^,!    MOVB ,!,!,!    MOVB ^,!,,!

MOVB ^,!,^,!   MOVB ^,!,!,!   MOVB !,!,,!    MOVB !,!,^,!

MOVB !,!,!,!   MOVW #^,^      MOVW #^,,!     MOVW #^,^,!

MOVW #^,!,!    MOVW ^,^       MOVW ^,,!      MOVW ^,^,!

MOVW ^,!,!     MOVW ,!,^      MOVW ^,!,^     MOVW !,!,^

MOVW ,!,,!     MOVW ,!,^,!    MOVW ,!,!,!    MOVW ^,!,,!

MOVW ^,!,^,!   MOVW ^,!,!,!   MOVW !,!,,!    MOVW !,!,^,!

MOVW !,!,!,!   MUL            NEG !,!        NEG ,!

NEG [!,!]      NEG [^,!]      NEG ^          NEG ^,!

NEGA           NEGB           NOP            ORAA !,!

ORAA #^        ORAA ,!        ORAA @^        ORAA [!,!]

ORAA [^,!]     ORAA ^         ORAA ^,!       ORAB !,!

ORAB #^        ORAB ,!        ORAB @^        ORAB [!,!]

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 41

ORAB [^,!]     ORAB ^         ORAB ^,!       ORCC #^

PSHA           PSHB           PSHC           PSHD

PSHX           PSHY           PULA           PULB

PULC           PULD           PULX           PULY

REV            REVW           ROL !,!        ROL ,!

ROL [!,!]      ROL [^,!]      ROL ^          ROL ^,!

ROLA           ROLB           ROR !,!        ROR ,!

ROR [!,!]      ROR [^,!]      ROR ^          ROR ^,!

RORA           RORB           RTC            RTI

RTS            SBA            SBCA !,!       SBCA #^

SBCA ,!        SBCA @^        SBCA [!,!]     SBCA [^,!]

SBCA ^         SBCA ^,!       SBCB !,!       SBCB #^

SBCB ,!        SBCB @^        SBCB [!,!]     SBCB [^,!]

SBCB ^         SBCB ^,!       SEC            SEI

SEV            SEX !,!        STAA !,!       STAA ,!

STAA @^        STAA [!,!]     STAA [^,!]     STAA ^

STAA ^,!       STAB !,!       STAB ,!        STAB @^

STAB [!,!]     STAB [^,!]     STAB ^         STAB ^,!

STD !,!        STD ,!         STD @^         STD [!,!]

STD [^,!]      STD ^          STD ^,!        STOP

STS !,!        STS ,!         STS @^         STS [!,!]

STS [^,!]      STS ^          STS ^,!        STX !,!

STX ,!         STX @^         STX [!,!]      STX [^,!]

STX ^          STX ^,!        STY !,!        STY ,!

STY @^         STY [!,!]      STY [^,!]      STY ^

STY ^,!        SUBA !,!       SUBA #^        SUBA ,!

SUBA @^        SUBA [!,!]     SUBA [^,!]     SUBA ^

SUBA ^,!       SUBB !,!       SUBB #^        SUBB ,!

SUBB @^        SUBB [!,!]     SUBB [^,!]     SUBB ^

SUBB ^,!       SUBD !,!       SUBD #^        SUBD ,!

SUBD @^        SUBD [!,!]     SUBD [^,!]     SUBD ^

SUBD ^,!       SWI            TAB            TAP

TBA            TBEQ !,^       TBL !,!        TBL ,!

TBL ^,!        TBNE !,^       TFR !,!        TPA

TRAP ^         TST !,!        TST ,!         TST [!,!]

TST [^,!]      TST ^          TST ^,!        TSTA

TSTB           TSX            TSY            TXS

TYS            WAI            WAV            WAVR

XGDX           XGDY

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 42

ADDRESSING MODES

INHERENT ADDRESSING

These instructions do not have operands and do not need access to

any memory locations.

Examples:

ABA

NOP

IMMEDIATE MODE (IMM)

These instructions have a single operand following the # symbol.

The operand is not a memory address. The operand is the value to

be used by the instruction. Omitting the # symbol will cause the

operand to be mistaken as direct mode.

Examples:

ADCA #$c5        ;IMM8

ADDD #$FDEA      ;IMM16

DIRECT MODE (DIR)

This mode can only access addresses in the range from $0000 to

$00FF. The operand given is assumed to be a memory address. If

the instruction requires an 8 bit value, the instruction reads in

the byte at the memory address given. If the instruction requires

a 16-bit value, the instruction reads in the byte at the address

given and stores it in the high order half. The next memory

address incrementally is read as the low order half.

Examples:

ADCA  $c8

CMPB  $45

EXTENDED ADDRESSING (EXT)

This addressing mode is used to access the full 64 kilobyte

memory. The operand of this instruction is a 16-bit memory

address.

Examples:

ADCA  $510a

EORA  $eb2c

INDEXED 5-BIT CONSTANT OFFSET (IDX)

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 43

In this addressing mode, two operands are given. The first is a

signed 4-bit number and the second is an index register. The

signed 4-bit number must have a value between -$10 and $0f. The

operation will proceed by reading the value in memory pointed to

by the index register offset with the first operand. The index

register can be either X, Y, PC, or SP. The index register is not

altered in this mode.

Examples:

EORA   -8,X

ETBL    2,X

INDEXED AUTO PRE/POST DECREMENT/INCREMENT (IDX)

This indexing mode allows the index register to be altered

before or after indexing takes place. There are two operands: a

signed value from 1 to 8, and the index register with a plus or

minus sign before or after it. If the sign is before the index

register, the first operand will be added or subtracted,

depending on its sign, from the index register. This new value

will be stored in the index register and used as the indexing

address. If the sign comes after the index register, the initial

value of the register will be used for indexing and the

increment/decrement will occur afterward. Only the X, Y, and SP

index registers can be used.

Examples:

LDAB      $8,x+   ; post increment x by 8

LDAB      $4,-x   ; pre decrement x by 4

INDEXED ACCUMULATOR OFFSET (IDX)

In this addressing mode, the contents of an accumulator are

added to the index register to determine the effective address.

The index register can be X, Y, SP, and PC and the accumulator

can be A, B, or D. The index register is not altered in this

mode.

Examples:

LDAB D,PC

LDS  B,SP

INDEXED 9-BIT CONSTANT OFFSET (IDX1)

Similar to 5-bit except it uses a full signed byte offset. The

first operand is a signed 8-bit number, with a value between

-$100 and $ff, and the second operand is an index register. The

operation will be carried out by reading in the value pointed to

be the index register offset by the first operand. The index

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 44

register can be either X, Y, PC, or SP. The index register is not

altered in this mode.

Examples:

EORA    -$10,X

INC     $c7,PC

INDEXED 16-BIT CONSTANT OFFSET (IDX2)

In this mode, a 16-bit offset is added to the index register, to

determine the proper memory address to access. This index mode

can access any address in memory offset from the index register.

It does not matter if the value is considered to be signed or

unsigned. The index register can be X, Y, PC, or SP. The index

register is not altered in this mode.

Examples:

INC     $6d17,SP

JSR     $CCFA,X

INDEXED 16-BIT CONSTANT INDIRECT ([IDX2])

In this addressing mode, there are two operands surrounded by

brackets. The first operand is the offset and the second is the

index register. The index register offset by the first operand is

the memory address of a pointer to the effective address. The

index register can be X, Y, SP, or PC. The index register is not

altered in this mode.

Examples:

JSR    [$d486,x]

LDAA   [$6e41,y]

INDEXED AUTO PRE/POST DECREMENT/INCREMENT (IDX)

This indexing mode allows the index register to be altered

before or after indexing takes place. There are two operands: a

signed value from 1 to 8, and the index register with a plus or

minus sign before or after it. If the sign is before the index

register, the first operand will be added or subtracted,

depending on its sign, from the index register. This new value

will be stored in the index register and used as the indexing

address. If the sign comes after the index register, the initial

value of the register will be used for indexing and the

increment/decrement will occur afterward. Only the X, Y, and SP

index registers can be used.

Examples:

LDAB      $8,x+   ; post increment x by 8

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 45

LDAB      $4,-x   ; pre decrement x by 4

INDEXED ACCUMULATOR OFFSET (IDX)

In this addressing mode, the contents of an accumulator are

added to the index register to determine the effective address.

The index register can be X, Y, SP, and PC and the accumulator

can be A, B, or D. The index register is not altered in this

mode.

Examples:

LDAB D,PC

LDS  B,SP

INDEXED D-INDIRECT ([D,IDX])

In this addressing mode, the contents of D are added to the

index register. This is the address of a pointer to a memory

location. There are two operands surrounded by brackets in this

mode. The first operand must always be D and the second is an

index register X, Y, SP, or PC. The index register is not altered

in this mode.

Examples:

LDS   [D,X]

LSR   [D,PC]

MULTIPLE ADDRESSING MODES

There are cases when multiple addressing modes can be used. With

the MOVE instruction both the source and the destination can have

different addressing modes. Bit manipulation instructions need

several operands, for the memory location, bits set, and address

to branch to. The second operand must be immediate, but the first

can be one of several addressing modes.

Examples:

MOVB  #$DF,$C600      ;IMM8-EXT

MOVW  $8EFA,D0,Y      ;EXT-IDX

BRCLR $01,SP+,$6F,$   ;IDX

BRSET -$10,X,$8A,$    ;IDX

IASM12.DOC Copyright P&E Microcomputer (1997)  Page - 46