# C&C Owners Fall Checkout List

WRITE A NOTE TO YOURSELF OF ALL THINGS LOOSENED OR COVERED IN THE FALL SO YOU CAN TIGHTEN OR UNCOVER IN THE SPRING.  LEAVE THE NOTE IN A CONSPICUOUS PLACE ON THE BOAT.

## Inside:

- Vacuum or clean all surfaces.

- Remove or prop up cushions.

- Prop open all doors and drawers.

- Remove VHF Radio.

- Remove Loran/GPS.

- Remove Autopilot.

- Spray electric terminals with WD-40.

- Remove Speedometer thru-hull.

- Clean and winterize bilge, remove bilge pump & service.

- Remove battery from clock.

## Freshwater System:

- Drain water tank and flush with anti-freeze.

- Pump anti-freeze through cold water system and run water through all faucets until anti-freeze appears.

- Pour anti-freeze into cooler drain and run pump.

## Head:

- Pump-out and flush holding tank thoroughly.

- Pour antifreeze through head into tank, make sure all the seacocks are open and drained.

- Condition toilet pump.

- Wash out vented loops.

## Engine:

- Top off fuel tank.

- Add stabilizer to fuel.

- Drain and replace engine oil and filter

- Replace transmission fluid.

- Clean water filter.

- Winterize external cooling system.

- Replace fuel filter.

- Run engine for a few minutes to run stabilized gas into engine.

- Open cylinder and exhaust manifold drain cocks.

- Loosen belts.

- Drain water pump.

- Remove impeller (Grease lightly and replace).

- Cover air inlet (Stuff with oil soaked rag).

- Tape exhaust outlet.

- Wipe down engine and vacuum engine compartment.

- Remove batteries.

- Remove the inner wires of all engine control cables from their outer sheaths. Inspect sheathing.

- Inspect all hoses for signs of softening.

## Outside:

- Clean deck and hull.

- Wax hull.

- Remove life lines if desired.

- Remove blocks from mast collar.

- Clean and wax stainless.

- Revarnish teak.

- Remove masthead instruments.

- Remove compass.

- Grease steering cables.

- Stuff a rag and tape over vent ports.

- Leave all drainage seacocks open.

- Cover boat.

## Propeller and Shaft:

- Check for cutlass bearing wear.

- Check propeller blade for misalignment.

- Inspect stainless steel shaft for crevice corrosion.

## Valves and Winches:

- Disassemble and regrease all winches.

- Disassemble all seacock plugs and regrease. (Check for scoring and leakage.)

Mast Wiring Terminal Block:

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Empty | Brown (two) | Green (x) | Yellow | Green | Yellow (x) |

Fuel Filter: 104500-55710  $6.88 @ Shumway

Oil Filter: 119660-35150 supercedes 124450-35110  $6.95 @ Shumway

Pier Point Marine:  315-483-6947

Notes:

Drain the seawater strainer (a plug in the bottom) drain the block (a faucet type fitting just above the oil  dipstick), drain the muffler.   Close up and replace all the plugs, remove the hose from the engine cooling

seacock, and place the end in a couple gallons of 50% antifreeze and water mix.  Start the engine and

run until all of the mix is gone.  Drain all of the mixture out again (as above) and you are done.