# C&C29 Spring Commissioning Checklist

## Sails and Rigging

 operate all sail furlers, inspect tuff-luff system

 check integrity of mast boot

 inspect turnbuckles and toggles for cracks and rust

 check all cotter pins and rings

 operate winches, grease as needed

 check all running rigging for chafe

 check all lines for proper whipping

 inspect sail seams, corners, grommets, battens, etc

 check all masthead hardware

## Safety

 check onboard safety gear

 check expiration dates on safety gear

 check expiration dates on fire extinguishers

 check expiration dates on first aid kit components

 inspect seizing wires of anchor shackles

 inspect lifelines for integrity

 pressure test propane appliances

 inspect cockpit drains for blockage

## Electrical System

 battery #1 voltage at 12.6VDC

 battery #2 voltage at 12.6VDC

 clean & tighten battery terminals

 operate circuit breakers/switches

 test lights & clean contacts or replace bulbs

| - side | - bow/anchor | - cabin |
| --- | --- | --- |
| - stern | - steaming | - compass |
| - strobe | - spreader |  |

## Electronics

 clean and lube electrical connector contacts

 monitor VHF radio transmit power

 clean and lube transducer O-rings

 tighten all instrument connections

## Plumbing

 test electric bilge pump

 clean bilge strainers as needed

 test pump float switch with water in bilge

 empty bilge with manual bilge pump

 exercise and lube head pumps and y-valves

 run and test freshwater pumps

 exercise and lubricate all seacocks

 flush antifreeze from freshwater hoses and pumps

 remove & clean engine & head  anti-siphon valves

## Engine At Rest

 check engine oil level

 check tranmission oil level

 check & adjust belt tension

 check safety wire on shaft setscrews

 check & tighten hose clamps

 inspect hoses for chaffing and bulging

 snug up bolts on engine & accessories

 exercise and open fuel tank petcocks

 inspect engine exhaust anodes

 inspect shaft anodes

 clean & lube articulating props

 clean raw water strainer

 exercise, lube, & open inlet seacock

## Engine Running

 monitor oil pressure

 exercise & lube throttle controls

 test transmission shifting

 look for oil leaks

 look for raw water/coolant leaks

 look for leaks around hoses & filters

 ensure exhaust water flows

 verify engine temperature after 15 min. run

 check alternator output at 13.5VDC

 monitor tachometer for smooth engine ops

 monitor packing gland drip rate 3 to 5 per min.

## Systems

 run heater or A/C; check for proper operation

 run watermaker; replace filters?; leak check

 run refrigerator; clean radiators; check ops

## Various

 lube and adjust steering

 clean and grease porthole gaskets

 touch up paint on bare wood

 ensure docking lines and fenders are on board

 list all onboard tools

 list all spare bulbs, fuses, hoses, clamps and tapes

 ensure docs, registration and licenses are on board

 make sure nav aids and charts are on board

 bottom paint inspect and recoat if needed

 cutless bearing inspect & replace if needed

 inspect stuffing box hoses & clamps

 clean, repair & wax gelcoat

 clean the stainless

 leak check windows, ports, & hatches; caulk