Steering Checklist

Right now is a great time to inspect your steering system. Don't wait until the spring, or until Murphy’s Law strikes, to check the condition of the steering system. A sailboat without its steering is like an out-of-control houseboat. Now, during the off-season, is a good time to check it out properly.

It’s not difficult to inspect a steering system. It takes an hour or so to do a thorough job. Mechanical aptitude is not the number-one credential. The most important tool you need are your eyes: If it doesn't look right, many times it isn't, and if you're not sure, call in the manufacturer of your steering system. If you don't have the time to do the inspection yourself, have the boatyard do it, or ask a sailing buddy

to help you in exchange for helping him or her. Generally, it's easy and straightforward. In fact, many people find that the most difficult part of the job is cleaning out the sail lockers in order to access the steering. But, hey, it's a good time to see if your emergency tiller is still there and to make sure it still fits.

Here is a 11-point checklist that will lead you through the process. While it was created by Edson, it will work for most brands and types of steering. (Numbers in parenthesis are Edson part numbers.)

Steering system inspection checklist

1. Steering wheel/shaft

Remove steering wheel for room to work.

Inspect wheel, key and snap ring.

Replace key if loose in keyway. (No. 684-250W)

Replace snap ring if there are signs of corrosion. (No. 960-A-660)

Remove compass and cylinder (follow manufacturer’s recommendations)

Replace compass bolts if corroded. (3 1/2" - No. 817-3.5, 1" - No. 817-1)

2. Engine controls

Inspect handles, levers, shafts, bushings.

Upgrade plastic handles to stainless; (Throttle - No. 963SB-55, Clutch - No. 963PT-55.

Replace Delrin bushings if deteriorated or stiff. (No. 960-A-125)

Inspect engine cables.

Replace cables if deteriorating or stiff. (No. 734-33, No. 735-64)

3. Wheel brake

Inspect brake. Tighten to determine if working properly. Visually inspect pads. Clean grease off of knurling.

Replace pads if worn or ineffective. (Brake Kit - No. 316-689)

Upgrade plastic knob to stainless (No. 960-A-91ST)

4. Steering chain/sprocket

Inspect chain for proper lubrication and free movement.

Replace if dry, corroded, or does not roll. (No. 886)

Inspect sprocket for broken, worn or bent teeth.

Replace sprocket if broken or bent. (No. 855-211)

5. Steering shaft bearings

Inspect condition of bearings by turning shaft and checking for play or resistance while chain is disconnected.

Replace bearings if stiff or excessive play. (No. 314-335)

6. Steering cables

Oil tissue and run along wire.

Inspect steering cable for signs of wear such as meat hooks or kinks.

Replace wires if there are any signs of wear. (No. 885)

Check cable tension; cable should deflect 1 inch per foot.

Tighten cables at the quadrant; take-up eyes if loose.

Inspect conduit (if used) for worn areas or tight bends.

Replace if worn through (No. 797-250.)

Lubricate conduit with Teflon grease if dry. (No. 827-3)

7. Cable sheaves/idler

Inspect idler plate and sheaves for corrosion or wear from misaligned cables. Look for metal dust under sheave.

Inspect sheave pins for excessive wear.

Replace idler, sheaves or pins if corroded or worn (see data sheet).

8. Steering cable alignment

Check that cables are centered in the groove of the sheaves and quadrant.

Adjust sheave placement to insure a fair wire lead.

(Correct cable alignment within the sheave system is required to insure longevity of the system.)

9. Quadrant / radial wheel

Inspect quadrant or radial for signs of wear or corrosion.

Inspect for cable wear along wire groove.

Inspect connection at rudder post for tightness.

Inspect rudder stop (Is it hitting supports on both sides?).

Replace quadrant if weakened by corrosion (see data sheet).

Replace rubber bumper on stop if missing. (No. 960-A-53)

Tighten all bolts clamping quadrant on rudder post.

Align quadrant or sheaves for fair wire runs.

10. Overall Inspection

Tighten all fasteners including pedestal bolts, wire rope, clamps and quadrant rudder post connections.

Inspect pedestal base for water leakage.

Seal with bedding compound if leaking.

11. Lubrication

Lubricate shaft roller bearings with Teflon grease. (No. 827-3)

Lubricate cable conduit with Teflon grease.

Oil wire rope, chain and sheave pins with No. 30 motor oil.

(For longevity of the steering system, proper lubrication is required.)

Print this checklist and take it with you to your boat.